Coda Heater PWM Info

Similar item to SFT-5528 (used by TSM in shop)

Note that duty cycle and frequency adjustments are interdependent. So, a single pot input may not be doable for heat control. See email 10/20/15 from Hans regarding his tests.

http://www.icstation.com/ne555-pulse-module-duty-cycle-frequency-adjustable-module-square-wave-rectangular-wave-signal-generator-p-2224.html

Video Link:

https://www.youtube.com/watch?v=97CU7WszMO0

Description:

1. Used as a square wave signal generator produces a square wave signal for the experimental development use.

2. Used to generate the square wave signal to drive a stepper motor drive.

3. Produce adjustable pulse for the MCU to use.

4. Generating adjustable pulse to control the associated circuits.

Parameter:

1. Size: 3.5CM * 3.6CM

2. The main chip: NE555

3. Input voltage: 5V-15V

Note: DC 5V power supply, the output current can about 15MA; 12V power supply, the output current can be about 35MA;

4. Input Current: ≥ 100MA

5. Output amplitude: 4.2V V-PP to 11.4V V-PP

(depends on the input voltage, the output amplitude is not the same)

6. Maximum output current:

≥ 15MA (5V power supply, V-PP greater than 50%)

≥ 35MA (12V power supply, V-PP greater than 50%)

Advantages: 1. Output with LED indication, there is no output LED quantity (low straightforward, high level LED off relatively low frequency LED flashes); 2. Output frequency range grade optional continuously adjustable output frequency; Low-frequency stall: 1Hz ~ 50Hz IF stall: 50Hz ~ 1kHz In the high-frequency stall: 1KHz ~ 10kHz High-frequency stall: 10kHz ~ 200kHz 3. the output duty cycle can fine-tune the duty cycle and frequency is not adjustable separately adjust the duty cycle will change the frequency 4. the output frequency is adjustable; Cycle T = 0.7*(Ra + 2*Rb)*C Ra, Rb:0-10K adjustable; LF gear C = 0.001UF; IF file C = 0.1UF; In the high-frequency file C = 1UF;, High-frequency gear C = 100UF, the waveform frequency buyers can own calculations. NOTE:Output duty cycle can fine-tune the duty cycle and frequency is not adjustable separately adjust the duty cycle will change the frequency.

Different waveforms are shown as below: